Tech Note

G3 CBW Cylinder Shell End Cracks at guide roller attachment bracket.

Repairing CBW shell end cracks on the G3 CBW's.

See crack pics below.

The guide roller bracket gets stress from transfer and also from guide rollers working against each other if installed on both sides of the same support ring.

We now only use one guide roller per support ring to prevent loading from an opposing roller on the other side of the same ring.

The part number for the doubler is 06 20689 (see the attached drawing).

First repair the shell end crack to make it watertight.

Then, grind off the weld at the end of the guide roller bracket.  Cut a notch out of the guide roller bracket to allow the doubler to slide between the guide roller bracket and the end of the shell face.  Weld as shown in the pictures.

GLL 03-29-18