Checking PC connections for Signal Light (Siglite).

- Can test if working by going to Drynet/Admin Tools/Siglite

If this test works than the PC is sending the signal.

- If not, check what port the cable is connected to.  Go to Control Panel/System/Ports and see what port is available either a USB to serial (Prolific port usually COM5 or 7) or Serial on Board (usually COM9).

- Make sure the siglite is looking for this port.  Go to Regedit:

Make sure ComPort matches what is shown in control panel.  Reboot if any change has been made.